# EuroBank

# exhibit 18

# Network Services Demarcation

# Table of Contents

1.	NETWORK SERVICES DEMARCATION	1

2.	CLARIFICATION OF SCOPE OF LAN SERVICES	2

2.1	Introduction	2

2.2	Scope of LAN services for passive components	2

2.3	Scope of LAN services for active components	3

- NETWORK SERVICES DEMARCATION

- CLARIFICATION OF SCOPE OF LAN SERVICES

  - Introduction

In general, the following components are needed for operating a LAN:

- passive components ( cabling, path panels )

- active components (switches/routers)

- network protocol services ( DNS, DHCP, WINS) needed for IP address assignment and name resolution

  - Scope of LAN services for passive components

a)	The connectivity from an end-user to the LAN environments consists of :

- in the office space: a patch cord A between the network card of the workstation/printer and the floor outlet

- in the office space: a fixed cable run B between the floor outlet and the patch panel C in the nearest wiring closet

- in the wiring closet: a patch panel C with a patch cord D to a switch S belonging to the access layer

- patch panels, patch cords and inter-floor cable runs (copper or fibre) between the layers of switches

b)	In scope are the following items and related operational and engineering services:

- the patch cord A

- patch cord D up to the access layer switch S

- registration of patch cords

- cords (copper or fibre) between the layers of switches within the same room

- registration of all cable runs & patches between the layers of switches within the same room

- registration of all patches related to links between layers of switches in separate rooms

- coordination activities with the stakeholders that take care of the out scope items

(“Registration” = keeping track of which patch panel positions are connected by a patch cord

  - Scope of LAN services for active components

A LAN design based on best practices results in a layered approach:

- an access layer to which the end-users are connected

- a distribution layer (optional, depends on size of the site)

o	which consolidates the access layer

o	to which non-user-equipment can be connected (server switches, external connection infrastructure)

o	which in absence of a core layer, assures connectivity to the trusted network outside the building

- a core layer (optional, depends on size and demarcation technique of the site)

o	assures connectivity to the trusted network outside the building

EuroBank has defined each of these layers and these have been combined and described in functional building blocks referred to as BLC cat 0…cat 4.

For smaller offices (e.g. BLC0 and 1) the demarcation boundary is the access switch (access layer in scope). For larger offices (e.g. BLC 2, 3 and 4) the boundary is the distribution switch (access layer and distribution layer in scope).

The core layer within a BLC 4 building only has layer 3 functionality, it has only routed links towards the distribution layer and is not part of the VTP domain of the BLC3 building blocks.